Operation Research

Part 1

Topic name: energy

| name | id | email |
| --- | --- | --- |
| Khaled walid | 222109 | Khaled222109@bue.edu.eg |

Research paper topic 1: Title: "1. A Mixed Integer Linear Programming Model for the Optimal Operation of a Network of Gas Oil Separation Plants (https://www.sciencedirect.com/science/article/pii/S0020025522001165)”

Problem Statement:

Running a network of Gas Oil Separation Plants (GOSPs) is a complicated process that requires making challenging decisions. These GOSPs are responsible for processing a combination of gas and oil that is extracted from wells. The objective is to efficiently process and transport these products throughout the network while taking into account various

Decision Variables:

- x_wgt: Volume of gas (cubic meters) processed at GOSP g from well w in period t (continuous)

- y_wgt: Volume of oil (barrels) processed at GOSP g from well w in period t (continuous)

- z_gwt: Volume of gas (cubic meters) transported from well w to GOSP g in period t (continuous)

- z_wot: Volume of oil (barrels) transported from well w to GOSP g in period t (continuous)

- v_gt: Volume of gas (cubic meters) stored at GOSP g in period t (continuous)

- u_gt: Volume of oil (barrels) stored at GOSP g in period t (continuous)

- δ_wt: Binary variable indicating if well w is producing in period t (1 - producing, 0 - not producing)

Objective Function:

Minimize total operating cost over the planning horizon (T):

Minimize Z = Σ_t Σ_w Σ_g (C_pg * x_wgt + C_po * y_wgt + C_tgw * z_gwt + C_tow * z_wot + C_stg * v_gt + C_sto * u_gt)

Constraints:

- Production Rate Constraints:

- Σ_g x_wgt <= P_wt * δ_wt (all w, t) - Ensures processed gas from well w in t doesn't exceed its production rate if producing (δ_wt = 1).

- Gas-Oil Ratio Constraints:

- y_wgt = x_wgt / GOR_w (all w, g, t) - Maintains the gas-oil ratio based on good properties.

- Processing Capacity Constraints:

- Σ_w x_wgt <= P_g (all g, t) - Ensures total processed gas at GOSP g doesn't exceed its capacity in period t.

- Σ_w y_wgt <= O_g (all g, t) - Ensures total processed oil at GOSP g doesn't exceed its capacity in period t.

- Storage Capacity Constraints:

- v_gt <= S_g (all g, t) - Limits gas storage at GOSP g not to exceed its capacity in period t.

- u_gt <= S_o (all g, t) - Limits oil storage at GOSP g not to exceed its capacity in period t.

- Material Balance Constraints:

- Σ_w z_gwt - x_wgt = 0 (all g, t) - Ensures all transported gas to GOSP g in t is processed.

- Σ_w z_wot - y_wgt = 0 (all g, t) - Ensures all transported oil to GOSP g in t is processed.

- v_gt-1 * v_(g,t-1) + Σ_w z_gwt = 0 (all g, t) - Ensures gas storage considers previous storage and incoming gas.

Research paper topic 2: Linear Programming for Scheduling Crude Oil Production (https://onepetro.org/JPT/article-pdf/10/07/51/2238336/spe-862-g.pdf)

The following material is dedicated to the application of linear programming (LP) in the context of scheduling the production of crude oil from multiple wells or fields in an optimal way. Let me explain the issue at hand:

Challenges:

- Production Decisions: Oil companies need to decide how much oil to extract from each well/field over a specific period. Multiple factors influence these decisions, including:

- Well/Field Capacity: Each well/field has limitations on the amount of oil it can produce daily or monthly.

- Reservoir Constraints: Extracting too much oil too quickly can deplete the reservoir faster than anticipated.

- Transportation Costs: Transporting oil from wells/fields to refineries incurs costs that vary depending on distance and infrastructure.

- Market Demand: The oil company needs to ensure enough oil is produced to meet refinery needs and fulfill sales contracts.

Objective:

The goal is to determine the optimal production schedule that maximizes the total profit for the oil company over the planning horizon. This involves balancing factors like:

- Oil Revenue: Selling the produced oil generates revenue for the company.

- Production Costs: Costs associated with extracting oil from each well/field.

- Transportation Costs: Costs of transporting oil to refineries.

Solution Approach:

The reference proposes using an LP model to represent the problem and find the optimal production schedule.

Decision Variables:

- y_jt: Amount of oil extracted from well/field j in period t (barrels) - continuous, non-negative

Objective Function:

Maximize total profit (Z) over the planning horizon:

Z = Σ_j Σ_t (p * y_jt - c_j * y_jt - t_j * y_jt)

where:

- p: The selling price of oil per barrel (constant)

- c_j: Production cost per barrel for well/field j (constant)

- t_j: Transportation cost per barrel for well/field j (constant)

Constraints:

The model incorporates several constraints to ensure the production schedule is feasible and respects limitations. These can include:

- Production Capacity Constraints:

- Σ_t (y_jt) <= A_j (all j) - This ensures the total amount of oil extracted from each well/field j (Σ_t) doesn't exceed its total capacity (A_j) over the planning horizon.

(Note: Alternatively, constraints can be written for each period to limit daily or monthly production).

- Reservoir Capacity Constraints (Optional):

- Σ_t (y_jt) <= R_j (all j) - This constraint can be included if there's a concern about exceeding the estimated recoverable reserves (R_j) for a well/field over the planning horizon.

- Market Demand Constraints:

- Σ_j Σ_t (y_jt) >= D_t (all t) - This ensures the total oil production (Σ_j Σ_t) across all wells/fields in each period (t) meets or exceeds the market demand (D_t).

- Transportation Network Constraints (Optional):

- These can be included if the model needs to consider limitations on pipeline capacities or restrictions on transporting oil from specific wells/fields to specific refineries.

Benefits:

By solving the LP model, the oil company can obtain a production schedule that:

- Maximizes overall profit by considering revenue, production costs, and transportation costs.

- Ensures production stays within well/field capacities and doesn't exceed estimated reserves (if considered).

- Meets or exceeds market demand for oil in each period.

A Study of the Relationship Between the Base Oil Price and the Profitability of Refinery Companies" doesn't directly focus on optimizing production scheduling, it provides valuable insights for formulating a linear programming problem related to base oil pricing. Here's a problem description based on the concepts from the reference:

Research paper topic

3: Optimization of Base Oil Price Using Linear Programming (https://archives.kdischool.ac.kr/bitstream/11125/32284/1/A%20Study%20of%20the%20relationship%20between%20the%20base%20oil%20price%20and%20profitability%20of%20refinery%20companies.pdf)

Problem:

The linear programming model for determining the ideal price for base oil can be formulated as follows:

Decision Variable:

B: Base oil price per unit (e.g., per barrel) - continuous, non-negative

Objective Function:

Maximize Domestic Refinery Profit:

Z = P_d * D - C_d * Q_d

where:

P_d: Domestic base oil selling price (B)

D: Domestic demand for base oil

C_d: Domestic production cost per unit of base oil

Q_d: Quantity of base oil produced domestically (dependent on B - higher price might incentivize more production)

Minimize Total Cost:

Z = P_d * Q_d + P_i * Q_i

where:

P_i: Import price per unit of base oil

Q_i: Quantity of base oil imported (dependent on B - a higher domestic price might lead to more imports)

Constraints:

Demand Satisfaction:

Q_d + Q_i >= D - This ensures total base oil availability (domestic production + imports) meets or exceeds domestic demand (D).

Price-Demand Relationship (Optional):

Q_d = a - b*B

where:

a and b are constants representing the initial demand and price sensitivity. A higher price (B) would lead to a decrease in demand (Q_d) due to market competition.

Non-Negative Constraint:

B >= 0 - The base oil price should not be negative.

Solving Base Oil Price Optimization with Simplex Method

Here's how we can solve the base oil price optimization problem using the simplex method, focusing on maximizing domestic refinery profit:

1. Setting Up the Initial Tableau:

| Basis | Q_d | Q_i | B | RHS |
| --- | --- | --- | --- | --- |
| C_d | -1 | 0 | C_d | -D |
| a | 1 | 0 | 0 | a |
| B | 0 | 0 | 1 | 0 |
| Z | D | -P_i | 0 | 0 |

drive_spreadsheetExport to Sheets

2. Choosing the Entering Variable:

We want to maximize profit (Z), so we look for the most negative coefficient in the Z row (excluding B, which is already basic). In this case, it's -P_i. This indicates that increasing Q_i (import quantity) will potentially increase profit.

3. Choosing the Leaving Variable:

Since we want to bring Q_i into the basis, we need to find the ratio of RHS to the corresponding coefficient in Q_i's column (excluding non-basic variables). The smaller this ratio (positive), the sooner we reach a non-negative value in that row.

- Ratio for C_d row: RHS (-D) / Q_i coefficient (0) = undefined (division by zero) - We can't use this row.

- Ratio for a row (if used): RHS (a) / Q_i coefficient (0) = undefined (division by zero) - We can't use this row (if the price-demand relationship is included).

4. Degeneracy Issue:

Both ratios for choosing the leaving variable are undefined. This indicates a degeneracy in the initial simplex tableau. There are a couple of ways to address this:

- Introduce an artificial variable: Add an artificial variable to one of the constraints (e.g., Demand Satisfaction) and adjust the objective function to penalize this artificial variable. This creates a new row with a positive coefficient for Q_i, allowing us to proceed with the simplex method. Once we have an optimal solution, we can check if the artificial variable has a value of zero. If it does, the solution is valid.

- Change constraint slightly: We can slightly modify one of the constraints (e.g., Demand Satisfaction) by adding a small positive constant (epsilon) to the RHS. This creates a non-zero coefficient for Q_i in that row, allowing us to proceed.

For simplicity, let's proceed assuming we address the degeneracy using the second approach (adding a small positive constant epsilon to the RHS of the Demand Satisfaction constraint).

5. Recalculating Tableau:

Assuming we added epsilon to the Demand Satisfaction constraint, the new tableau might look like this:

| Basis | Q_d | Q_i | B | RHS |
| --- | --- | --- | --- | --- |
| C_d | -1 | 0 | C_d | -D + epsilon |
| a | 1 | 0 | 0 | a |
| B | 0 | 0 | 1 | 0 |
| Z | D | -P_i | 0 | 0 |

Now, the ratio for the C_d row becomes:

- Ratio for C_d row: (RHS -D + epsilon) / Q_i coefficient (0) = positive (since epsilon is positive)

Since this is the only positive ratio, we choose the C_d row as the pivot row. Q_d is already the basic variable in this row.

6. Performing Pivot Operation:

We perform the pivot operation on the C_d row with Q_d as the pivot variable. This will bring Q_i into the basis and eliminate Q_d from the non-basic variables.

7. Subsequent Iterations:

We continue iteratively by choosing the entering variable (most negative coefficient in Z excluding B) and the leaving variable (based on the ratio of RHS to corresponding coefficient in entering variable's column). We perform pivot operations until we reach an optimal solution where all coefficients in the Z row are non-negative and the chosen objective function (maximizing domestic refinery profit) is achieved.

Conclusion:

By solving the system of equations through the simplex method iterations, you can determine the base oil price (B)